Step 1: connect RS232 to USB cable to USB port on windows OS computer and RS232 port on battery.

Step 2: open Device Manager and find out the COM port ID for RS232 to USB converter. (In my case, it’s COM4)

Step 3: Execute BMS Tool software and select the correct Serial Port and click Open button.

Step 4: If everything is alright. It should be Normal on the Realtime Monitoring Page.

Step 5: Go to System Config page.

Step 6: In System Config page click on Read Button in Inverter Protocol Section.

Step 7: Select Relevant CAN Protocol or RS485 Protocol as required then enter Password (Password: Pz#168178). Then click write button.

Step 8: Connect battery to Victron Inverter via CAN port. A CAN to VE Can converting cable will be required. Please follow the instructions on following link to make one if required. https://www.victronenergy.com/live/battery_compatibility:can-bus_bms-cable#introduction